# Network Monitoring & Security Operations Lab

Author: Saketh Thokala

Project Type: Cybersecurity & Network Security Lab

Platform: Cisco Packet Tracer

# Overview

This project demonstrates an enterprise-style network security and monitoring environment built in Cisco Packet Tracer. It covers VLAN segmentation, inter-VLAN routing, SSH remote management, switch port security, unused-port hardening, ACL enforcement, centralized Syslog monitoring, and security verification.

# Objectives

Build a multi-VLAN enterprise-style network.

Configure inter-VLAN routing using R2 router-on-a-stick.

Configure a dedicated management VLAN.

Secure access ports with sticky MAC port security.

Disable unused switch ports.

Configure SSH version 2 for secure remote management.

Enforce traffic restrictions using an extended ACL.

Centralize device logs using Syslog.

Verify connectivity and security controls.

# Network Devices

| Device | Type | Role |
| --- | --- | --- |
| R1-EDGE | Cisco 2911 | Simulated edge/upstream router |
| R2-CORE | Cisco 2911 | Inter-VLAN routing, ACL and Syslog |
| SW1-HQ | Cisco 2960 | Core/distribution switching |
| SW2-HQ | Cisco 2960 | HR/Finance access |
| SW3-ACCESS | Cisco 2960 | Sales/IT access |
| SW4-ACCESS | Cisco 2960 | Server/Management access |
| SRV1-MONITORING | Server-PT | Central Syslog server |
| PC1–PC10 | PC-PT | HR, Finance, Sales, IT and Admin endpoints |

# VLAN and IP Addressing Plan

| VLAN | Department | Network | Gateway |
| --- | --- | --- | --- |
| 10 | HR | 10.10.10.0/24 | 10.10.10.1 |
| 20 | Finance | 10.10.20.0/24 | 10.10.20.1 |
| 30 | Sales | 10.10.30.0/24 | 10.10.30.1 |
| 40 | IT | 10.10.40.0/24 | 10.10.40.1 |
| 50 | Servers | 10.10.50.0/24 | 10.10.50.1 |
| 99 | Management | 10.10.99.0/24 | 10.10.99.1 |

# Security Controls

## VLAN Segmentation

Separate VLANs are used for HR, Finance, Sales, IT, Servers, and Management.

## Inter-VLAN Routing

R2-CORE provides the default gateway for each VLAN through G0/0 subinterfaces.

## SSH Remote Management

R2 and all switches use SSH version 2 with local authentication. Credentials are excluded.

## Port Security

Active access ports use one sticky MAC address with violation mode restrict.

## Unused-Port Hardening

Unused switch interfaces were administratively shut down.

## ACL Enforcement

HR-RESTRICTION denies HR-to-Finance traffic while permitting HR traffic to other destinations.

## Centralized Syslog

R2 and switches send logs to SRV1-MONITORING at 10.10.50.10 using UDP 514.

# Verification Commands

show ip interface brief
show vlan brief
show interfaces trunk
show ip access-lists HR-RESTRICTION
show logging
show ip ssh
show port-security interface

# Evidence Screenshots

01-enterprise-topology.png

02-management-vlan-verification.png

03-sw2-management-verification.png

04-sw3-management-verification.png

05-sw4-management-verification.png

06-ssh-remote-management.png

07-port-security-baseline.png

08-acl-enforcement.png

09-acl-policy-verification.png

10-syslog-r2-verification.png

11-central-syslog-verification.png

12-syslog-server-configuration.png

13-final-r2-security-verification.png

# Skills Demonstrated

Cisco Packet Tracer; Cisco IOS; TCP/IP; IPv4; VLANs; trunking; inter-VLAN routing; extended ACLs; SSH; port security; network hardening; Syslog monitoring; troubleshooting; security verification; technical documentation.

Note: This is a simulated Cisco Packet Tracer lab intended to demonstrate practical networking and defensive security concepts.